Anisotropic effects on the thermophysical properties of U3Si2 fuel: An atomic scale study

Ericmoore Jossou1, Md. Jahidur Rahman1, Dotun Oladimeji2, Benjamin Beeler3, Barbara Szpunar2, Jerzy Szpunar1

1 Department of Mechanical Engineering, College of Engineering, University of Saskatchewan, 57 Campus Drive, Saskatoon, S7N 5A9, Saskatchewan, Canada.

2 Department of Physics and Engineering Physics, College of Art and Science, University of Saskatchewan, 116 Science Place, Saskatoon, S7N 5E2, Saskatchewan, Canada

3Idaho National Laboratory, Idaho Falls, ID, 83415, USA

# Abstract

As a result of the renewed interest in uranium silicide compounds as a candidate for nuclear reactor fuels, there is a need for extensive knowledge of their thermophysical properties as a function of temperature. In this work, we calculated the thermophysical properties of the U3Si2 compound within the framework of molecular dynamics (MD) using a semi-empirical modified Embedded-Atom Method (MEAM) potential. Thermal expansion, thermal conductivity, heat capacity, and elastic properties are presented as a function of temperature. The thermal conductivity of U3Si2 increases with temperature due to the electronic contribution while the phonon contribution decreases with increasing temperature. The phonon contribution to the thermal conductivity at 300 K is 1.37 W/m-K and 1.36 W/m-k using non-equilibrium molecular dynamics (NEMD) and equilibrium molecular dynamics (EMD), respectively. The inherent anisotropic thermophysical properties can be use in phase field models for simulating the thermal gradient and centerline temperature under normal reactor and accident conditions.

Keywords: Uranium silicide; ATF; molecular dynamics; thermophysical; MEAM.

*Corresponding author. E-mail: ericmoore.jossou@usask.ca

1. Introduction

Uranium dioxide (UO2) is the standard fuel in today’s Light Water Reactors (LWRs). The nuclear accident at Fukushima in 2011 provided motivation for fuel vendors, in collaboration with national and international regulatory agencies, to consider an alternative fuel that can withstand the loss of coolant for a considerably longer time in the case of an accident. Alternate fuel types must maintain high safety and reliability during normal operation, and it would be preferable to not make major changes to the present design of the LWRs in the near future [1,2]. Binary uranium-silicon compounds such as U3Si2 have attracted a lot of attention as a nuclear fuel material because of some favorable properties compared to UO2. For instance, U3Si2 has a higher thermal conductivity and higher fissile material density[3]. The enhanced heat removal capabilities make uranium silicide fuel a promising potential fuel for generation IV reactors. However, until recently there has been limited study on the thermal properties of uranium silicide [4] compared to other properties such as oxidation behavior [5,6], structural information [7,8], and radiation resistance [9,10].

According to experiments [7,8], U3Si2 has a tetragonal crystal structure (space group 127) with U occupying 2a and 4h Wyckoff positions while Si occupies the 4g site as shown in Figure 1. Using a particle swarming optimization code, Noordhoek et al., predicted a new U3Si2 phase with space group number 2 which has not been experimentally synthesized [11].

Figure 1: Unit cell of ordered uranium silicide containing 2 formula units of U3Si2 (10 atoms). (Color scheme: U=grey, Si=blue)

Due to the growing interest in Accident Tolerant Fuels (ATfs) in the last few years, some work has been conducted on U-Si alloys focused on U3Si2. Density functional theory (DFT) calculations by Wang et al. provided detailed data on the electronic structure [12] and work by Middleburgh et al. modeled the formation energies of point defects [13]. White et al. developed a feasible powder metallurgy method of preparation and measured the thermophysical properties of U3Si2 [3]. More recently, Obbard et al. used in-situ neutron diffraction to collect the temperature dependence of lattice parameters and the lattice thermal expansion coefficient was calculated. They observed an anisotropy in lattice expansion at ~1000 oC, corroborating the dilatometry measurement of White et al. [3, 14]. . Our recent work within the framework of DFT explores the mechanism of oxidation from an atomistic perspective [15], while work by Wood et al., experimentally showed the oxidation kinetics in the presence of air and steam [16–18]. Furthermore, these authors investigated the possibility of reducing oxidation by alloying of U3Si2 with aluminium due to formation of an Al2O3 passivation layer which can prevent further oxidation [19]. Recent in-situ TEM observation of Xe ion irradiated U3Si2 indicates that, at normal reactor operation conditions, uranium silicide is stable and no radiation-induced amorphization was reported [21]. In a similar experiment using Kr ions, Miao et al. noticed that U3Si2 is oxidized forming a mixture of nanocrystalline UO2 and a Si-enriched amorphous phase, raising concern for radiation assisted corrosion issues [10].

Beyond the properties of pristine U3Si2, it is essential to understand the underlying behavior in an extreme environment typical for a nuclear reactor. The radiation-induced defects and fission products in nuclear reactor fuel result in microstructural changes which in turn alters the fuel’s thermophysical properties [20]. This has a significant effect on the thermal conductivity of the fuel matrix, which decreases as radiation dose increases due to the contribution from phonon scattering. Our recent work within the framework of DFT explores the mechanism of oxidation from an atomistic perspective [15], while work by Wood et al., experimentally showed the oxidation kinetics in the presence of air and steam [16–18]. Furthermore, these authors investigated the possibility of reducing oxidation by alloying of U3Si2 with aluminium due to formation of an Al2O3 passivation layer which can prevent further oxidation [19].  Such behavior is difficult and expensive to quantitatively determine from experiment.

Due to the limitation and cost of irradiation experiments, Beeler et al. developed a MEAM potential to model the formation of  point defects, formation energy and thermophysical properties of U3Si2 [22]. The interatomic potential developed in Beeler’s work will be used in this work to calculate various thermophysical properties of U3Si2.

The thermophysical properties such as specific heat, the coefficient of thermal expansion, density and the thermal conductivity of the fuel as a function of temperature are important and must be known in order to effectively model and predict fuel behavior under normal and off-normal irradiation conditions. In this work, we calculated these properties and evaluate the results obtained by comparing them with the experimental data available in the existing literature.

2. Computational details

2.1 Phonon assisted thermal conductivity calculation

Molecular Dynamics (MD) simulations, employing a set of interatomic potentials for U3Si2 derived previously by Beeler et al. [22] are carried out using the Large-scale Atomic/Molecular Massively Parallel Simulator (LAMMPS) [23]. In this model the potential energy Ei, of an atom i with respect to all other atoms has two components which are: i) a pair potential description of each system and ii) an angular dependent many-body embedding function contribution. The detailed formalism of the modified many-body embedded atom method (MEAM) is well described in references [24,25].

The phonon assisted thermal conductivity (kph) of a material is determined by several scattering mechanisms which include isotopic scattering, point defect scattering, three-phonon scattering and grain boundary scattering to name just a few. The value of kph can be calculated in different ways [26] including the relaxation time approximation [27], Callaway model [28], Slack model [26], Non-Equilibrium Molecular Dynamics (NEMD) [26], Equilibrium Molecular Dynamics (EMD) with Green-Kubo formalism [29,30] and by solving Boltzmann transport equation iteratively [31].  In this work, we calculated kph using the MD-based methods and the electronic thermal conductivity (kel) was approximated using the Wiedemann–Franz law relationship [32]. In this work, the calculation of thermal conductivity was done using both EMD and NEMD methods for comparison.

2.1.1 Equilibrium Molecular Dynamics method

In the EMD approach, the Green-Kubo method relates the equilibrium fluctuations of the heat current vector, S, to the thermal conductivity tensor via the fluctuation-dissipation theorem. The phonon assisted thermal conductivity (kph) in the α-direction is given by Eq.1 [29,30]

| kph,α=1KBVT20∞⟨SαtSα0⟩dt, α = x, y or z direction | (1) |
| --- | --- |

where KB is the Boltzmann constant, t is time, T is temperature, ⟨SαtSα0⟩  is the ensemble average of Sα in the α-direction, usually called the heat current autocorrelation function (HCACF). Sα denotes the heat current in the α-direction which can be computed by Eq.2 [33,34]:

| Sα=iEivi+ 12ij>i(Fij(vi+vj)rij, | (2) |
| --- | --- |

where Ei and vi are the total energy and velocity of atom i, respectively, Fij is the force exerted by atom j on atom i, and rijis the instantaneous and equilibrium interatomic separation vectors between atoms i and j. The EMD simulations were performed using the LAMMPS code [23]. The starting structures of size 14×14×16 were first run for 4,000,000 time steps using an isothermal-isobaric (NPT) ensemble to allow the volume to expand, and then equilibrated under a constant volume and energy ensemble (NVE) for 4,000,000 time steps at temperatures of interest. A time step of 0.25 fs was deemed suitable to maintain energy conservation.  To avoid statistical noise during heat flux collection, NVE was run for an additional 2,000,000 time steps. Correlation functions were evaluated from the heat current time series.

2.1.2 Non-Equilibrium Molecular Dynamics method

In the NEMD simulations, the systems were initially equilibrated using an NPT ensemble with a Nosé-Hoover thermostat and barostat with a damping time of 0.1 ps and 0.5 ps respectively at zero external pressure. Equilibration is achieved when there is convergence in total energy and volume of the system.

These structures are equilibrated for 2 fs for temperatures between 300 K and 1800 K at 15 K intervals with the thermophysical properties (lattice parameter and enthalpy) obtained from averages taken over the final 0.25 fs of the simulation.

The equilibrated structures were also used for thermal conductivity simulations using the non-equilibrium MD method. A time step of 1 fs was used throughout the simulation. In the NEMD method [35], kinetic energy can be transferred between atoms from one region of the supercell to another. Here, we used a rectangular supercell (slab) of five different lengths including 100, 125, 150 and 175 unit cells in the z-direction. The supercell is split into several bins with a constant heat flux, Jz, generated by heat exchange of atoms between the hot and cold region, respectively, as shown by the schematic illustration in Figure 2 (inset). The temperature gradient, ∆a, is established between the two extreme temperatures due to the kinetic energy exchange. Eventually, a steady state in thermal gradient is reached where Jz is equal and opposite to the kinetic energy exchange so that the temperature profile remains constant. In Figure 2 (main), a steady state temperature distribution, along the direction of heat transfer, is plotted for the largest system (5 × 5 × 175 unit cells) studied with the hot region at both ends and the cold region at the middle of U3Si2 This shows that the temperature at the center of the supercell is higher than that at the edge. The slope of the profile is the temperature gradient of the system, ∆a. Temperature control can be simulated by addition or subtraction of the non-translational kinetic energies in the heat bath such that momentum is conserved.

The phonon thermal conductivity, kph, is computed using Eq. 3:

| kph=-Jz∆a, | (3) |
| --- | --- |

where the heat flux, Jz is given by Eq. 4:

| Jz = KEz2tLxLy, | (4) |
| --- | --- |

KEz is the amount of transferred kinetic energy calculated over all exchanges during the simulation time t in the z-direction. Lx and Ly are the supercell lengths of the area through which the heat is transferred. The temperature gradient, ∆a, is given by Eq. 5:

| ∆a=∆T∆Lz, | (5) |
| --- | --- |

where T is the temperature in Kelvin and Lz is the length of the supercell in the z-direction.

Figure 2: A steady-state temperature profile for a 5 × 5 × 175 unit cell system with the hot region at both ends and the cold region at the middle of U3Si2. The inset is a schematic illustration of the NEMD simulation setup used to study thermal conductivity of U3Si2. (Color scheme: U=grey, Si=blue).

2.2 Electronic thermal conductivity calculation

The electronic thermal conductivity (kel) can be calculated from electrical conductivity (σ) of a material using Wiedemann-Franz law [32]:

| kel = σLT, | (6) |
| --- | --- |

where L is the Lorenz number which equals L = 2.44 × 10−8 W Ω K−2.

The experimentally measurable quantity is the electrical resistivity (ρ) which is related to σ by:

| ρ=1σ=1στ, | (7) |
| --- | --- |

where τ is the electronic relaxation time and στ is calculated using the Semi-Classical Boltzmann Theory with the relaxation time approximation and the rigid band approximation as implemented in the BoltzTraP code [36] . The full tensor of σ is calculated from the conductivity distributions:

| σ T;μ=1Vσ(ε)-∂fμ(T;ε)∂εdε, | (8) |
| --- | --- |

Where f is the Fermi-Dirac distribution function, ɛ is the band energy, µ is the Fermi level and σαβ(ϵ) is defined as:

| σαβϵ=1Ni, kσαβi, kδ(ε-εi, k)dε, | (9) |
| --- | --- |

Where k is a set of k-points elements in reciprocal space, N is the number of k-points sampled and i, is the band index and σαβi, k is given by:

| σαβi, k=e2τi, kvα(i, k)vβ(i, k), | (10) |
| --- | --- |

Where e is the elementary charge, vα is the group velocity of the electrons. Based on Eqs. 6–10, the full tensor of σ is calculated using τi, k and the electronic band structure data which is calculated within the framework of  DFT as implemented in the Quantum ESPRESSO code [37] with a dense k-points mesh of 20×20×18. We assume that τi, k is constant and does not depend on the band index i or direction k, which has been shown to be a satisfactory assumption for many metals [38]. At 300 K, τ is evaluated by equating the experimental ρ of 1.05×10-6 Ωm given by Shimizu [39] to στ calculated from the BoltzTraP code.

3. Results and discussion

3.1 Lattice parameter and thermal expansion

As a preliminary step, we calculate the lattice parameter ‘a’ and ‘c’ as a function of temperature. Not only is it required for all simulations, but also the thermal expansion coefficient α is a simple characteristic that depends directly on the anharmonic terms in the potential. Therefore, it gives a quick indication of how appropriate the interatomic potential is for studying thermal conductivity. The total energies, lattice parameters and volumes were determined by averaging over the last 50 ps of the simulations. The linear thermal expansion coefficient (LTEC) is then given by the relationship:

| αp =1L∂L∂Tp | (11) |
| --- | --- |

where L is the lattice parameter. The derivative of the lattice parameter with respect to temperature, ∂L∂T is calculated by fitting a straight line to the lattice parameter at a specific temperature, T, and the points within T±15 K on either side (See supplementary material for detailed plots from which lattice gradients are extrapolated) in order to reduce numerical fluctuations. The calculated lattice parameters a, c and cell volume at 0 K are presented in Table 1. Our results are in good agreement with the work by Beeler et al. given that the same interatomic potential is used in this work. There is a variation between our MD results and previous DFT + U and experimental measurements. For instance, a comparison with DFT + U calculations from Noordhoek et al. showed that a is underestimated by ~2.1 % and c overestimated by ~4 %.

Table 1: Structural properties (lattice parameter a, c (Å) and volume (Å3)) of U3Si2 calculated at 0 K using MEAM U-Si potential and compared to DFT + U calculations as well as experiment.

|  | a | c | Volume/atom | c/a |
| --- | --- | --- | --- | --- |
| This work (MD) | 7.078 | 4.064 | 20.355 | 0.574 |
| Beeler (MD) et al. [22] | 7.078 | 4.064 | 20.355 | 0.574 |
| Noordhoek (DFT+U) et al. [11] | 7.230 | 3.891 | 20.339 | 0.538 |
| Wang (DFT+U) et al. [12] | 7.32 | 3.89 | 20.82 | 0.53 |
| Jossou (DFT+U) et al. [15] | 7.21 | 3.86 | 20.07 | 0.54 |
| Zachariasen (Expt.) [7] | 7.3151 | 3.8925 | 20.80 | 0.53 |
| Obbard (Expt.) et al. [14] | 7.324 | 3.882 | 20.832 | 0.530 |

The average lattice parameters a, c and cell volume v as a function of temperature obtained from the MD simulation for U3Si2 are shown in Figure 3a-c. Experimental values are also plotted in the range from 298 K to 1872 K. The lattice parameter a and cell volume are underestimated while c is overestimated but it is clear that the MD results follow the experimental trend. Some disagreement with experiment is understandable since the interatomic potential model was not fitted to this data. In Figure 3a-c the behavior of the lattice parameters and cell volume can be seen to be almost linear in the range of 298-1800 K.

Figure 3d-f shows thermal expansion ΔL/L0 as function of temperature where L is lattice parameters a, c and cell volume v for U3Si2 structure. There is a subtle change in the thermal gradient at temperature points which coincidentally marks the onset of change in the LTEC calculated for U3Si2 in Fig. 4. The underestimation of the LTEC in comparison with experimental data is predictable as the individual components of the thermal expansion are also underestimated, especially after ~500 K.

(f)

(e)

(d)

(a)

(b)

(c)

Figure 3: (a-c) Shows variation of lattice parameter and cell volume (d-f) Thermal strain in lattice parameters and in cell volume as a function of temperature. The experimental lattice constants and lattice strain data were obtained from Refs. [14].

We compared the LTEC computed from the volumetric strain (αv) with experimental measurements and the previous MD result. A fairly good agreement is observed with the data calculated here and the work of Loch et al. [40] and Shimizu [39] especially at temperatures above 900 K, while underestimation is noticed in comparison to the recent measurement by White et al. [3] and Taylor et al. [41]. The average LTEC from 300 to 1200 K is 11.81× 10-6 K-1, in excellent agreement with the result of Beeler et al. [21].

(a)

(b)

Figure 4: (a) Linear thermal expansion coefficients of U3Si2 calculated from volumetric strain (b) anisotropy in linear thermal expansion coefficient in a and c direction as a function of temperature. The experimental linear thermal expansion coefficient data were obtained from Refs. [3, 39-41].

Beeler et al. [21] calculated an average LTEC but did not report unique a/c expansion coefficients. Additionally, this work utilizes a more refined temperature sampling, which allows for a more intricate analysis of the nature of anisotropic thermal expansion. This work is performed in order to compare with the experimental results from Obbard et al.  [14]. We looked at the directional dependence of LTEC and it is interesting to note that there are three transition regimes in the a direction: at 500, 1300 and 1700 K, with a LTEC of 9.86, 12.54 and 13.86 × 10-6 K-1, respectively. As shown in Figure 4b, there is a transition in the c direction at 600 K and a comparison with a recent study where a transition point is established at ~1300 K [14] shows agreement with one of the transition points observed in our calculation. These differences might be due to the nature of the MEAM potential since it was not developed for calculating thermophysical properties [22]. It is also well known that thermal expansion can be made up of lattice, electronic, magnetic and vacancy/interstitial components, depending upon the material and temperature range. The MD model does not account for all of these factors, and the electronic contribution could be especially important given the metallic nature of U3Si2 [42].

To further investigate the observed anisotropy, we compute the c/a ratio which is a measure of the tetragonality of the structure. The change in c/a with temperature is important, because it has a direct influence on the evolution of the LTEC.

Figure 5: The temperature dependence of the c/a ratio of U3Si2.  The experimental c/a data were derived from Ref. [14].

The non-uniform expansion of the a and c lattice parameters is responsible for the change in the c/a ratio with temperature as shown in Figure 5, which in turn results in the anisotropic nature of the LTEC in U3Si2. The c/a ratio is overestimated in our calculation but follows the experimental trend none the less.

3.2 Enthalpy and Specific heat

In this work we determine the enthalpy increment (increase in enthalpy with respect to standard temperature and pressure as a function of temperature, using the relationship HT-H(298K). Just like the lattice parameter, the enthalpy increment increases approximately linearly with temperature as shown in Figure 6a. The first derivative of the enthalpy increment with respect to temperature is used to calculate the specific heat capacity at constant pressure due to phonon scattering, Cph, (including both harmonic and anharmonic contributions) using the following relationship,

| Cph =1n∂H∂Tp | (12) |
| --- | --- |

where n is the number of moles. The first derivative of the enthalpy was calculated by fitting a straight line to the enthalpy (See supplementary material for detailed plots from which enthalpy gradients are extrapolated) at a given temperature T and the data at T±15 K on either side. The molar heat capacity of a metal is the summation of the dilatational, harmonic and anharmonic lattice (phonon), and electronic terms. The total molar heat capacity,  Cp is estimated by addition of the Cph and the electronic heat capacity, Cel given by:

| Cel(T)=γT, | (7) |
| --- | --- |

where γ is the electronic specific heat coefficient, experimentally determined as 52.0 mJ/mol-K2 [3] and estimated to be 13.2 mJ/mol-K2 from DFT+U calculations [43], and T is the temperature in Kelvin. The calculated Cph values agreed with the Dulong Petit Law value above the Debye temperature. The sum of Cph and Cel components of heat capacity give the total heat capacity, Cp, shown as the red and black data points in Figure 6b. In comparison to previous work[3,44], there is a good agreement from 300 to 900 K especially with the experimentally determined γ. The electronic contribution to heat capacity is quite significant since the Dulong-Petit limit for phonon specific contribution is ~125 J/mol-K. This is consistent with work by Beeler and coworkers who determined Cph at 400 K to be 134 J/mol-K [22] which agrees with recently measured value by Antonio et al. at 300 K as 149 J/mol-K [4].

(a)

(b)

Figure 6: (a) The change in enthalpy relative to the enthalpy at 300 K (b) molar heat capacity at constant pressure as a function of temperature. The experimental heat capacity at constant pressure data were obtained from Refs. [3, 44].

3.3 Elastic constants

Elasticity describes the physical and internal changes of solid material under the application of stress and strain. The elastic coefficients and moduli can be also linked to the atomic bonding, deformation characteristics and mechanical and structural stability. To investigate the mechanical stability of the intermetallic U3Si2, we computed the elastic constants from the ground-state elastic tensor using structures optimized at 0 and 300 K using the scripts provided with LAMMPS code [23]. The calculated elastic constants of a single crystal of U3Si2 are listed in Table 2 along with the reported values from DFT calculations [11,12,43] and experiment [45]. For tetragonal symmetry, there are six independent elastic constants, that is, C11, C12, C13, C33, C44 and C66. The calculated elastic constants of U3Si2 fulfil the Born criteria for mechanical stability [46]. Therefore, it can be said that U3Si2 is mechanically stable at ambient conditions.

While DFT calculations are believed to be more accurate than MD simulations, their high computational cost strongly limits the length and time scales used in such calculations. The bulk and shear moduli here are estimated using the Hill’s model which is an average of the Voigt and Reuss bounds [47] while Beeler et al. used only the Voigt bound, hence the difference in bulk modulus. As shown in Table 2, there is a general softening in the elastic constant as we increased the temperature from 0 to 300 K, and a look at the experimental measurements which are done at room temperature shows that Young's modulus is overestimated in our MD calculation by ~15–20% at 300 K, and ~4–9% at 0 K, respectively. There are various deviations in the calculated Bulk moduli in comparison to the measured values of 68.3 and 64.3 GPa, which are from the different sets of U3Si2 samples prepared at Los Alamos National Laboratory [45]. It is also important to mention that theoretical calculations assume a single crystal during simulations while experimental measurements were conducted on polycrystalline U3Si2 samples with the possible presence of porosity, grain boundaries and microcracks, which might account for the general overestimation in DFT and MD results. The external defects can significantly reduce the magnitude of the elastic properties [48].

The elastic and thermophysical properties of solids are related through the Debye model. Furthermore, the Debye temperature, θD is related to many physical properties such as specific heat, elastic constants, and melting point, Tm [49]. The mean sound velocity, vm, in polycrystalline materials is approximated by [50]:

| vm=132vt3+1vl3-13, | (13) |
| --- | --- |

where the transverse velocity vt=Gρ and the longitudinal velocity vl=3B+4G3ρ. The Debye temperature is then calculated using the relationship [46]:

| θD=ħk3n4πNAρM12vm, | (14) |
| --- | --- |

where ħ is the Planck's constant, k is Boltzmann’s constant, NA is Avogadro's number, n is the number of atoms per formula unit, and ρ is the density.

The calculated elastic constants can be used to estimate the Tm of solids based on a correlation derived by Fine et al. from studying many cubic and tetragonal structures [51]. This is given by:

| Tm=553K+5.91K/GPa(C11)±300, | (15) |
| --- | --- |

Using Eqs. 13–15, the polycrystalline properties of U3Si2 are calculated and are presented in Table 3. These results are also compared to recent DFT result from Wang et al. [12]. The calculated value of the Tm from data at 0 K and 300 K are 1795.46±300 and 1785.65±300 K, respectively. Furthermore, DFT work by Szpunar et al. reports Tm for paramagnetic and ferromagnetic U3Si2 phases as 1796 and 1250 K respectively [52]. These values are reasonably close to the experimental value of 1938 K [53] and the MD result of 1775 K. This relation provides a quick method for estimating the Tm using the elastic constants calculated in this work. The calculated value of the Debye temperature of U3Si2 agrees well with the reported theoretical value of 280.92 K [4]. Although it is overestimated by approximately 76 (81) K in comparison to a recent experimental measurement of 200 K, this is still considered excellent agreement.

Table 2: Elastic constants (GPa), Elastic moduli (GPa) of U3Si2 at 0 and 300 K. U3Si2 specimens labelled a= set A (samples1 – 5) and b= set B (samples 6 – 7) were prepared from different feedstocks [45].

|  | C11 | C33 | C12 | C13 | C44 | C66 | G | B | E | ν |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MEAM at 0K (This work) | 210.23 | 160.20 | 178.90 | 65.75 | 103.10 | 35.40 | 51.09 | 126.67 | 135.13 | 0.32 |
| MEAM at 0K [22] | 210.2 | 160.2 | 178.9 | 65.8 | 103.1 | 35.4 |  | 134 | – | – |
| MEAM at 300K (This work) | 208.57 | 145.79 | 144.9 | 67.01 | 90.99 | 47.1 | 60.00 | 118.70 | 154.04 | 0.28 |
| DFT at 0K [11] | 149 | 139 | 49 | 48 | 63 | 46 | 53 | 81 | 130 | 0.23 |
| DFT at 0K [12] | 167.26 | 205.31 | 45.63 | 50.34 | 67.49 | 74.09 | 67.68 | 92.01 | 163.06 | 0.20 |
| DFT at 0K [43] | 155 | 142 | 47 | 50 | 65 | 46 | 57 | 83 | 139 | 0.22 |
| Experiment [45] | 137.9 | – | – | – | 53.75 | – | – | 68.3±0.5a 64.3±0.3b | 130.4±0.5a 122.9±2.4b | 0.182±0.003a 0.179±0.007b |

Table 3: Theoretical sound velocities (m/s), Debye and melting temperatures (K) of U3Si2 predicted using the elastic properties at 0 and 300 K

| Temperature (K) | Vl | Vt | Vm | θD | Tm |
| --- | --- | --- | --- | --- | --- |
| 0 | 4174.17 | 2295.05 | 2558.19 | 276.19 | 1795.46±300 |
| 300 | 4125.83 | 2338.76 | 2600.55 | 280.76 | 1785.65±300 |
| DFT at 0 [12] | 3865.04 | 2355.32 | 2601.53 | 280.92 | 1541.51±300 |

3.4 Thermal conductivity

The thermal conductivity of nuclear fuel pellets is an important thermophysical property in nuclear reactors. It is argued in the literature that the high thermal conductivity of metallic systems is mainly contributed by the electrons (el) while the phonon (ph) contribution is small, and hence neglected in some cases [32]. Phonon scattering occurs in three phonon interactions, which result from anharmonicities in the interatomic potential. The other phonon scattering center which contributes to thermal conductivity includes impurities, grain boundaries or local distortions of the lattice. These contributions are not accounted for in this work, as their effects are expected to be statistically insignificant in U3Si2.

The total thermal conductivity of metallic nuclear fuel is determined by the combination of the contribution to the thermal conductivity both from electronic and phonon transport as follows:

| ktotal=kel+ kph, | (16) |
| --- | --- |

The  kel is the electronic contribution to the thermal conductivity has been calculated in this work using the Wiedemann-Franz law [32]. We have calculated the kph of U3Si2 at temperatures from 300 to 1500 K by both the NEMD and the EMD methods. The size of the supercell has a considerable effect on the thermal conductivity calculated using both methods. Therefore, rigorous convergence testing for a finite simulation cell size was carried out for the two methods.

In the EMD approach, the convergence of  kph with respect to the HCACF integral and finite simulation cell size are key sensitivity tests that are needed to achieve converged values of thermal conductivity. The HCACF integral is directly proportional to the thermal conductivity (see Eq. 4). Convergence in HCACF is achieved by varying the correlation length (truncation time) until it decayed to zero. The normalized HCACF (main figure) and its integral (inset) at 300 and 1200 K are shown in Figure 7a. The HCACF integral converged after 0.90 ns, hence we specify the converged value of the HCACF integral by averaging its value between times of 0.90 and 1.25 ns (the shaded region of the inset).

(a)

(b)

The number of phonons needed to capture the ph-ph scattering in the bulk material is dependent on the simulation cell size. This effect is reduced by increasing the simulation cell length until the thermal conductivity reaches a size independent value. In this work, we investigated system sizes of 8×8×10, 10×10×12, 12×12×14, 14×14×16 and 16×16×18. We have computed the thermal conductivity of U3Si2 using supercells containing 12000, 20160, 31360, and 46080 atoms, all at a temperature of 300 K. These systems correspond to lengths Lz  of 4.68, 5.46, 6.24, and 7.02 nm respectively. In Figure 7b we show the computed thermal conductivity obtained by direct integration for 1.25 ns of the HCACF for different choices of simulation cell. We see that the 12000 atom (3.90 nm) system results in the lower value of 0.73 W/mK, compared to the simulations in a larger system size of 46080 atoms (7.02 nm). However, it is apparent from Figure 7b that the results for kph are well converged by 31360 atoms (6.24 nm). The same finite size convergence tests have been used in a previous work [54].

(a)

(b)

Figure 7: (a) Normalized heat current autocorrelation function (body) and its integral (inset) for U3Si2 at 300 and 1200 K. The HCACFs are normalized by their initial values. The shaded region in the inset indicates the time range over which the HCACF integral is averaged to predict the phonon assisted thermal conductivity kph. Note that the time scale of the inset is 50 times that of the body (b) Thermal conductivity plotted against lateral simulation box dimensions at 300 K. Results converged at supercell length of 6.24 nm.

In the NEMD, it has been shown in the previous study that the finite size of the system can limit phonon mean free path, hence underestimating the thermal conductivity. Furthermore, it is well known that the supercell dimension in the Lx and Ly directions have a small effect on the thermal conductivity, but changes along the Lz are more significant [54].

Figure 8: The variation of the reciprocal of thermal conductivity as a function of the reciprocal of supercell length for perfect U3Si2 at a range of temperatures. The y-intercept indicates the reciprocal of bulk thermal conductivity.

The linear relationship between 1k and 1Lz  is well described in Ref. [35] and plotted in Figure 8a for the perfect U3Si2 structure for a number of temperatures with supercell sizes of 5×5×100, 5×5×125, 5×5×150, and 5×5×175. The y-intercept is used to determine the thermal conductivity of the bulk U3Si2 as shown in Figure 8a.

Having computed the thermal conductivity of U3Si2 using the NEMD and the EMD method, we are now in a position to directly compare the results. For the NEMD at 300 K, extrapolation to 1Lz = 0 yields a value of kph of 1.37 W/mK. The EMD method gives a result of 1.36 W/mK for a 14×14×16 system at 300 K. The predicted thermal conductivities from the two methods are plotted against temperature from 300 – 1500 K in Figure 9a (main). Unlike the Boltzmann Transport Equation (Lattice dynamics and MD) prediction methods, there are no major assumptions inherent to the EMD and NEMD methods other than the choice of the interatomic potential and the damping parameters, which are consistent here. The predicted thermal conductivity values from the EMD and NEMD methods are in good agreement, generally within about 6% (except at temperatures of 900 and 1200 K) where the predictions are within 10%. Previous work by others have also found that EMD and NEMD thermal conductivity predictions are consistent [54,55]. Our MD results compare reasonably well with recent work by Antonio et al., who predicted a kph from 0 – 350 K, with a value of 1.99 W/mK at 300 K, which is overestimated compared to our EMD (1.36 W/mK) and NEMD (1.37 W/mK) results, respectively [4].

This provides evidence that either method can be applied to compute bulk thermal conductivity in perfect crystalline U3Si2. It is important to note that the many-body MEAM potential used here is not developed with a special focus on thermophysical properties of U3Si2.

Figure 9: kph of U3Si2 using EMD and NEMD approach over the temperature range of 300 to 1500 K.

We further explored the directional dependence of  kph as shown in Figure 9a (inset), which shows the kzkxvariation as a function of temperature. The  kph  exhibits significant directional anisotropy. In order to track the trend, we fitted the data to a linear equation which points to a moderate decrease in  kzkx  from 0.73 at 300 K and 0.39 at 1500 K. This anisotropic behavior may be attributed to differences in group velocity, anharmonicity, and scattering phase space along the different directions [56]. It is well known that the longitudinal sound velocity ( vl) and the transverse sound velocity ( vt) are primarily harmonic parameters while their ratio vlvt is an anharmonic quantity which depends on the ratio between the axial and shear rigidities of interatomic bonds [57]. We find that the value of vlvt is 1.76 which is a deviation from isotropic behavior. This implies that the anisotropy in the  kph of U3Si2 should be ascribed to its anisotropic sound velocities amongst other factors that have not been explored in this work.

In metallic system like U3Si2, it has been shown that at the low temperature i.e. T ≪θD, only phonon scattering, ph–ph, plays a role in the thermal conductivity calculation [32,58] while in the T ≫θD regime, el–el interaction is the main contribution, hence it is not surprising to have low kph at 300 K given the fact that the θD has been determined to be ~200 K [3,4,59]. Low kph contribution means that as temperature increases, degradation in phonon-based thermal conductivity is not a big concern in U3Si2 fuel, since kel is the dominant component of  ktotal and increase with increasing temperature.

In the calculation of kel, the value σ/τ is determined from Eq. (8). Figure 10a is a plot of σ/τ of electrons along different crystallographic directions in U3Si2 over the temperature range of 300 to 1500 K. We observed that overall, the value σ/τ shows a very weak temperature dependence with a 10% change over the entire range of 300 to 1500 K. A plot of  kel in x and z crystallographic directions is shown in Figure 10 b. The average kel values in the different directions fall within the spread of kx and kz data. Our result slightly overestimates kel in comparison to recent work by Srinivasu et al. [60]. This is not surprising since they used electrical resistivity data from Miyadai et al. [61] which is different from the value reported by Shimizu used in this work. Ranasinghe et al. [62] used the value from Shimizu, but the kel is slightly underestimated with reference to this work due to the fact that temperature dependent values of τ are used in the calculations.

We obtain slightly higher  ktotal values compared to recent theoretical [63] and experimental [3] data as shown in Figure 10c.  Similar trends have been reported for the thermal conductivity values of α-uranium where a similar method was used [64]. Furthermore, values reported by Shimizu are lower compared to our data which might be due to the defects in the structure and method of measurement [39].

(a)

(b)

(c)

Figure 10: (a) Plot of στ of electrons in U3Si2 (b) directional dependence kel and (c) ktotal  over the temperature range of 300 to 1500 K. The experimental and theoretical thermal conductivity data were obtained from Refs. [3, 39, 60, 62, 63].

4. Conclusion

In summary, we have performed a systematic molecular dynamics study to investigate the anisotropic, temperature-dependent thermophysical properties of U3Si2 using a MEAM interatomic potential and first principles calculations. Based on the equilibrated structure of U3Si2, we systematically obtain the values of thermal expansion coefficient, bulk modulus, and heat capacities at different temperatures, which are in good agreement with corresponding experimental measurements. A comparison of calculated molar heat capacity with experimental data shows reasonable agreement with experiments. Due to the anisotropy in lattice expansion with temperature, we noticed an anisotropic behavior in the in-plane and cross-plane linear thermal expansion coefficient which has also been experimentally observed. The agreement between our calculations and experiments also proves the validity of our methods and model, and the effectiveness of the MD method employed in this work.

In addition, we also calculated the lattice assisted thermal conductivity of U3Si2. Both the EMD (Green–Kubo) and the direct (NEMD) methods have been utilized to determine the phonon-based thermal conductivity. The Green–Kubo method was also used for all predictions of the directional dependence of the in-plane and cross-plane kph, for which application of the direct method is impractical. The electronic thermal conductivity was determined from first principles band structure calculations and the Wiedemann-Franz law. The electronic contribution to the thermal conductivity in U3Si2 was confirmed to be the predominant component at temperatures of interest.  Unlike other metallic fuel such as UB2 where there is significant kph [65], U3Si2 has a kph less than 2 W/mK, which means that above the Debye temperature, the thermal gradient is mainly driven by kel which increases with temperature. Therefore, ‘kph drag’ on the ktotal at reactor’s operating temperature is insignificant compared to UB2 or similar fuel type.  The results also indicate an anisotropic behavior of the termal conductivity in the x and z directions.  The total thermal conductivity calculated is slightly overestimated compared to a recent experiment by White et al. [3], but generally shows excellent agreement across the entire temperature range.

Overall, this work serves as a first step towards understanding the complex anisotropic behavior of the thermophysical properties of metallic U3Si2 nuclear fuel. The predicted results can be incorporated in mesoscale models such as Phase Field to understand how thermal gradient and centerline temperature evolve during under normal operation and accident condition.

Acknowledgement

The authors gratefully acknowledge Compute Canada (West-Grid) and the University of Saskatchewan's Research Cluster (Plato) for the generous amount of CPU time. We also recognize the financial support from the Natural Sciences and Engineering Research Council of Canada (NSERC) and International Dean’s Scholarship.

References

[1]	S.J. Zinkle, K.A. Terrani, J.C. Gehin, L.J. Ott, L.L. Snead, Accident tolerant fuels for LWRs: A perspective, J. Nucl. Mater. 448 (2014) 374–379.

[2]	I. Charit, Accident Tolerant Nuclear Fuels and Cladding Materials, JOM. 70 (2018) 173–175.

[3]	J.T. White, A.T. Nelson, J.T. Dunwoody, D.D. Byler, D.J. Safarik, K.J. McClellan, Thermophysical properties of U3Si2 to 1773 K, J. Nucl. Mater. 464 (2015) 275–280.

[4]	D.J. Antonio, K. Shrestha, J.M. Harp, C.A. Adkins, Y. Zhang, J. Carmack, K. Gofryk, Thermal and transport properties of U3Si2, J. Nucl. Mater. 508 (2018) 154–158.

[5]	K. Johnson, V. Ström, J. Wallenius, D.A. Lopes, Oxidation of accident tolerant fuel candidates, J. Nucl. Sci. Technol. 54 (2017) 280–286.

[6]	E. Sooby Wood, J.T. White, A.T. Nelson, Oxidation behavior of U-Si compounds in air from 25 to 1000 C, J. Nucl. Mater. 484 (2017) 245–257.

[7]	W.H. Zachariasen, Crystal Chemical Studies of the 5f-Series of Elements . VIII. Crystal Structure Studies of Uranium Silicides and of CeSi2 , NpSi2 and PuSi2, Acta Crystallogr., 2 (1949), p. 94.

[8]	K. Remschnig, T. Le Bihan, H. Noël, P. Rogl, Structural chemistry and magnetic behavior of binary uranium silicides, J. Solid State Chem. 97 (1992) 391–399.

[9]	Y.S. Kim, G.L. Hofman, J. Rest, A.B. Robinson, Temperature and dose dependence of fission-gas-bubble swelling in U3Si2, J. Nucl. Mater. 389 (2009) 443–449.

[10]	Y. Miao, J. Harp, K. Mo, S. Bhattacharya, P. Baldo, A.M. Yacout, In-situ TEM ion irradiation investigations on U3Si2 at LWR temperatures, J. Nucl. Mater. 484 (2017) 168–173.

[11]	M.J. Noordhoek, T.M. Besmann, D. Andersson, S.C. Middleburgh, A. Chernatynskiy, Phase equilibria in the U-Si system from first-principles calculations, J. Nucl. Mater. 479 (2016) 216–223.

[12]	T. Wang, N. Qiu, X. Wen, Y. Tian, J. He, K. Luo, X. Zha, Y. Zhou, Q. Huang, J. Lang, S. Du, First-principles investigations on the electronic structures of U3Si2, J. Nucl. Mater. 469 (2016) 194–199.

[13]	S.C. Middleburgh, R.W. Grimes, E.J. Lahoda, C.R. Stanek, D.A. Andersson, Non-stoichiometry in U3Si2, J. Nucl. Mater. 482 (2016) 300–305.

[14]	E.G. Obbard, K.D. Johnson, P.A. Burr, D.A. Lopes, D.J. Gregg, K.-D. Liss, G. Griffiths, N. Scales, S.C. Middleburgh, Anisotropy in the thermal expansion of uranium silicide measured by neutron diffraction, J. Nucl. Mater. 508 (2018) 516–520.

[15]	E. Jossou, U. Eduok, N.Y. Dzade, B. Szpunar, J.A. Szpunar, Oxidation behavior of U3Si2: an experimental and first principles investigation, Phys. Chem. Chem. Phys. 20 (2018) 4708–4720.

[16]	E. Sooby Wood, J.T. White, A.T. Nelson, Oxidation behavior of U-Si compounds in air from 25 to 1000 C, J. Nucl. Mater. 484 (2017) 245–257.

[17]	A.T. Nelson, A. Migdisov, E.S. Wood, C.J. Grote, U3Si2 behavior in H2O environments: Part II, pressurized water with controlled redox chemistry, J. Nucl. Mater. 500 (2018) 81–91.

[18]	E.S. Wood, J.T. White, C.J. Grote, A.T. Nelson, U3Si2 behavior in H2O: Part I, flowing steam and the effect of hydrogen, J. Nucl. Mater. 501 (2018) 404–412.

[19]	E.S. Wood, J.T. White, A.T. Nelson, The effect of aluminum additions on the oxidation resistance of U3Si2, J. Nucl. Mater. 489 (2017) 84–90.

[20]	M.R. Tonks, X.-Y. Liu, D. Andersson, D. Perez, A. Chernatynskiy, G. Pastore, C.R. Stanek, R. Williamson, Development of a multiscale thermal conductivity model for fission gas in UO2, J. Nucl. Mater. 469 (2016) 89–98.

[21]	Y. Miao, J. Harp, K. Mo, Y. Soo, S. Zhu, A.M. Yacout, Microstructure investigations of U 3 Si 2 implanted by high-energy Xe ions at 600 C, J. Nucl. Mater. 503 (2018) 314–322.

[22]	B. Beeler, M. Baskes, D. Andersson, M.W.D. Cooper, Y. Zhang, A modified Embedded-Atom Method interatomic potential for uranium-silicide, J. Nucl. Mater. 495 (2017) 267–276.

[23]	S. Plimpton, Fast Parallel Algorithms for Short-Range Molecular Dynamics, J. Comput. Phys. 117 (1995) 1–19.

[24]	M.I. Baskes, J.S. Nelson, A.F. Wright, Semiempirical modified embedded-atom potentials for silicon and germanium, Phys. Rev. B. 40 (1989) 6085–6100. doi:10.1103/PhysRevB.40.6085.

[25]	M.I. Baskes, Modified embedded-atom potentials for cubic materials and impurities, Phys. Rev. B. 46 (1992) 2727–2742.

[26]	D.T. Morelli, G.A. Slack, High Lattice Thermal Conductivity Solids BT  - High Thermal Conductivity Materials, in: S.L. Shindé, J.S. Goela (Eds.), Springer New York, New York, NY, 2006: pp. 37–68.

[27]	H. Gzyl, Integration of the boltzmann equation in the relaxation time approximation, J. Stat. Phys. 29 (1982) 617–622.

[28]	J. Callaway, Model for lattice Thermal Conductivity at Low Temperatures, Phys. Rev. 113 (1959) 1046–1051.

[29]	R. Kubo, Statistical-Mechanical Theory of Irreversible Processes. I. General Theory and Simple Applications to Magnetic and Conduction Problems, J. Phys. Soc. Japan. 12 (1957) 570–586.

[30]	M.S. Green, M.R. Processes, Markoff Random Processes and the Statistical Mechanics of Time-Dependent Phenomena, 1281 (2000).

[31]	M. Omini, A. Sparavigna, An Iterative Approach to the Phonon Boltzmann Equation in the Theory of An iterative approach to the phonon Boltzmann equation in the theory of thermal conductivity, 4526 (2018).

[32]	J.M. Ziman, Electrons and Phonons: The Theory of Transport Phenomena in Solids, Oxford University Press, Oxford, 2001.

[33]	M. Kaviany, I. Introduction, Phonon Transport in Molecular Dynamics Simulations : Formulation and Thermal Conductivity Prediction, 39 (2006) 169–255.

[34]	Z. Liang, A. Jain, A.J.H. Mcgaughey, P. Keblinski, Molecular simulations and lattice dynamics determination of Stillinger-Weber GaN thermal conductivity, 125104 (2015) 1–5.

[35]	M.J.R. and B.S. and J.A. Szpunar, The induced anisotropy in thermal conductivity of thorium dioxide and cerium dioxide, Mater. Res. Express.

[36]	G.K.H. Madsen, D.J. Singh, BoltzTraP. A code for calculating band-structure dependent quantities, Comput. Phys. Commun. 175 (2006) 67–71.

[37]	P. Giannozzi, S. Baroni, N. Bonini, M. Calandra, R. Car, C. Cavazzoni, D. Ceresoli, G.L. Chiarotti, M. Cococcioni, I. Dabo, A.D. Corso, S. de Gironcoli, S. Fabris, G. Fratesi, R. Gebauer, U. Gerstmann, C. Gougoussis, A. Kokalj, M. Lazzeri, L. Martin-Samos, N. Marzari, F. Mauri, R. Mazzarello, S. Paolini, A. Pasquarello, L. Paulatto, C. Sbraccia, S. Scandolo, G. Sclauzero, A.P. Seitsonen, A. Smogunov, P. Umari, R.M. Wentzcovitch, QUANTUM ESPRESSO: A modular and open-source software project for quantum simulations of materials, J. Phys. Condens. Matter. 21 (2009) 395502.

[38]	W.W. Schulz, P.B. Allen, N. Trivedi, Hall coefficient of cubic metals, Phys. Rev. B. 45 (1992) 10886–10890.

[39]	H. Shimizu, The Properties and Irradiation Behavior of U3Si2, Tech. Rep. NAASR-10621, Atomics International, July 1965.

[40]	L.D. Loch, G.B. Engle, M.J. Snyder, W.H. Duckworth, Survey of Refractory Uranium Compounds, Tech. Rep. BMI-1124, Battelle Memorial Institute, August 1956.

[41]	K.M. Taylor, C.H. McMurtry, Synthesis and Fabrication of Refractory Uranium Compounds, Tech. Rep. ORO-400, Carborundum Company, February 1961.

[42]	Characterization of Nuclear Materials, (n.d.).

[43]	D. Chattaraj, C. Majumder, Structural, electronic, elastic, vibrational and thermodynamic properties of U3Si2: A comprehensive study using DFT, J. Alloys Compd. 732 (2018) 160–166.

[44]	O. Knacke, O. Kubaschewski, K. Hesselmann, Tables, Thermochem. Prop. Inorg. Subst. (1991) 1–2541.

[45]	U. Carvajal-Nunez, T.A. Saleh, J.T. White, B. Maiorov, A.T. Nelson, Determination of elastic properties of polycrystalline U3Si2 using resonant ultrasound spectroscopy, J. Nucl. Mater. 498 (2018) 438–444.

[46]	Z. Wu, E. Zhao, H. Xiang, X. Hao, X. Liu, J. Meng, Crystal structures and elastic properties of superhard IrN2 and IrN3 from first principles, Phys. Rev. B. 76 (2007) 54115.

[47]	R. Hill, The Elastic Behavior of a Crystalline Aggregate, Proc. Phys. Soc. Sect. A. 65 (1952) 349.

[48]	M.J. Rahman, B. Szpunar, J.A. Szpunar, Effect of fission generated defects and porosity on thermo-mechanical properties of thorium dioxide, J. Nucl. Mater. 510 (2018) 19–26.

[49]	M.A. Ghebouli, B. Ghebouli, M. Fatmi, First-principles calculations on structural, elastic, electronic, optical and thermal properties of CsPbCl3 perovskite, Phys. B Condens. Matter. 406 (2011) 1837–1843.

[50]	O. Anderson, J. Phys. Chem. Solids, 24 (1963) 909–917..

[51]	M.J. Mehl, B.M. Klein, D.A. Papaconstantopoulos, First principles calculations of elastic properties of metals, 1 (1994) 1–26.

[52]	B. Szpunar, L. Malakkal, E. Jossou, J.A. Szpunar, First Principles Investigations of Alternative Nuclear Fuels BT  - Energy Materials 2017, in: X. Liu, Z. Liu, K. Brinkman, S. Das, S. Dryepondt, J.W. Fergus, Z. Guo, M. Han, J.A. Hawk, T. Horita, P. Hosemann, J. Li, E. Olivetti, A. Pandey, R.B. Rebak, I. Roy, C. Shang, J. Zhang (Eds.), Springer International Publishing, Cham, 2017: pp. 367–376.

[53]	A. Berche, C. Rado, O. Rapaud, C. Guéneau, J. Rogez, Thermodynamic study of the U–Si system, J. Nucl. Mater. 389 (2009) 101–107.

[54]	P.K. Schelling, S.R. Phillpot, P. Keblinski, Comparison of atomic-level simulation methods for computing thermal conductivity, Phys. Rev. B. 65 (2002) 144306.

[55]	J.E. Turney, E.S. Landry, A.J.H. McGaughey, C.H. Amon, Predicting phonon properties and thermal conductivity from anharmonic lattice dynamics calculations and molecular dynamics simulations, Phys. Rev. B. 79 (2009) 64301.

[56]	H. Peng, N. Kioussis, D.A. Stewart, H. Peng, N. Kioussis, D.A. Stewart, Anisotropic lattice thermal conductivity in chiral tellurium from first principles Anisotropic lattice thermal conductivity in chiral tellurium from first principles, 251904 (2015).

[57]	H. Shao, X. Tan, G.-Q. Liu, J. Jiang, H. Jiang, A first-principles study on the phonon transport in layered BiCuOSe, Sci. Rep. 6 (2016) 21035.

[58]	A.H. Wilson, The theory of metals / by A. H. Wilson, Cambridge Uni. Press, Cambridge, England, 1953.

[59]	J.T. White, A.T. Nelson, D.D. Byler, D.J. Safarik, J.T. Dunwoody, K.J. McClellan, Thermophysical properties of U3Si5 to 1773K, J. Nucl. Mater. 456 (2015) 442–448.

[60]	K. Srinivasu, B. Modak, T.K. Ghanty, Electronic structure and thermophysical properties of U3Si2: A systematic first principle study, J. Nucl. Mater. 510 (2018) 360–365.

[61]	T. Miyadai, H. Mori, T. Oguchi, Y. Tazuke, H. Amitsuka, T. Kuwai, Y. Miyako, Magnetic and electrical properties of the U-Si system (part II), J. Magn. Magn. Mater. 104–107 (1992) 47–48.

[62]	J.I. Ranasinghe, E. Jossou, L. Malakkal, B. Szpunar, J.A. Szpunar, Study on Radial Temperature Distribution of Aluminum Dispersed Nuclear Fuels: U3O8-Al, U3Si2-Al, and UN-Al, J. Nucl. Eng. Radiat. Sci. 4 (2018) 31020–31027.

[63]	Y. Zhang, A.D.R. Andersson, A thermal conductivity model for U-­Si compounds, United States, 2017.

[64]	S. Zhou, R. Jacobs, W. Xie, E. Tea, C. Hin, D. Morgan, Combined ab initio and empirical model of the thermal conductivity of uranium, uranium-zirconium, and uranium-molybdenum, Phys. Rev. Mater. 2 (2018) 83401.

[65]	E. Jossou, L. Malakkal, B. Szpunar, D. Oladimeji, J.A. Szpunar, A first principles study of the electronic structure, elastic and thermal properties of UB2, J. Nucl. Mater. 490 (2017) 41–48.